Phase 1: OS Imaging & Initial Setup

- OS: Raspberry Pi OS (64-bit).

- Settings (Gear Icon): * Enable SSH.

  - Set Username/Password (Remember these!).

  - Configure Wi-Fi (This is for the initial setup so you can download OpenCV).

- Boot & Connect: Plug in your Pi, find its IP on your router, and SSH into it from your laptop.

Phase 2: Create the Hotspot

The command you have is correct, but we need to ensure the Pi doesn't try to be a "Wi-Fi Client" and a "Hotspot" at the same time on the same interface, which can cause conflicts.

- Create the Hotspot:

Bash

sudo nmcli device wifi hotspot ssid <YourSSID> password <YourPassword>

- Verify Name: Run nmcli connection show. The name of your connection might be Hotspot.

- Auto-Connect: ```bash sudo nmcli connection modify "Hotspot" connection.autoconnect yes

- Static IP for Pi: It is best if the Pi always has the same IP as the "Router."

Bash

sudo nmcli connection modify "Hotspot" ipv4.addresses 192.168.50.1/24 ipv4.method shared

Phase 3: Dependencies & Hardware

- Update & Install:

Bash

sudo apt update

sudo apt install python3-opencv libqt5gui5 libqt5test5 -y

- Webcam Check: Plug in your webcam. Run ls /dev/video*. If you see /dev/video0, you are good to go.

- Permissions: (Crucial for your script to read the DHCP leases)

Bash

sudo chmod 644 /var/lib/misc/dnsmasq.leases

Phase 4: The Systemd Service

Your service file looks great, but let's add After=NetworkManager.service to ensure the hotspot is actually up before the Python script tries to find a "connected guest IP."

Create file: sudo nano /etc/systemd/system/mypicamera.service

Paste this refined version:

Ini, TOML

[Unit]

Description=Boat Control Python Script

# Wait for NetworkManager to finish starting the hotspot

After=NetworkManager.service network-online.target

Wants=network-online.target

[Service]

# Root is required to read dnsmasq.leases

User=root

Environment=QT_QPA_PLATFORM=offscreen

# Ensure this path matches where you put your script!

ExecStart=/usr/bin/python3 /home/pi/your_script.py

WorkingDirectory=/home/pi/

StandardOutput=inherit

StandardError=inherit

Restart=always

RestartSec=5

[Install]

WantedBy=multi-user.target

Activate it:

Bash

sudo systemctl daemon-reload

sudo systemctl enable mypicamera.service

sudo systemctl start mypicamera.service

Once you have finished downloading your updates and OpenCV, you should tell the Pi to "forget" your home Wi-Fi or prioritize the Hotspot.

- Run: nmcli connection show

- Find your home Wi-Fi name.

- Run: sudo nmcli connection modify "YourHomeWiFi" connection.autoconnect no

This ensures that when the Pi boots up in the middle of a lake (or anywhere without your home router), it doesn't waste time searching for a signal and goes straight into Hotspot Mode.

Final "Pre-Flight" Checklist

Before you disconnect the monitor and keyboard for the last time:

- Check Autoconnect: nmcli con show (The "Hotspot" line should be green).

- Check Service: systemctl is-enabled mypicamera.service (Should return enabled).

- Check Lease File: ls -l /var/lib/misc/dnsmasq.leases (Ensure it exists).

- Test Reboot: Run sudo reboot. Wait 60 seconds. See if the Wi-Fi network appears on your laptop. If it does, you are officially "Automated."

Important Troubleshooting Tips

- The "0" Index: Sometimes the Pi assigns the webcam to /dev/video0, but if you have multiple camera drivers loaded, it might be 1 or 2. If your script fails, try cv2.VideoCapture(0, cv2.CAP_V4L2).

- Read/Write Permissions: If your script saves images, ensure the folder has the correct permissions: chmod +777 /path/to/your/folder.

- Virtual Environments: If you used a venv on your laptop, you must point the ExecStart in the service file to the python executable inside that folder (e.g., /home/pi/myenv/bin/python3).

Since you won't see the OpenCV windows, you can check if your script is actually seeing your laptop by reading the logs: journalctl -u mypicamera.service -f This will show you your print(f"Targeting Computer at: {UDP_IP}") messages in real-time!

Troubleshooting "No Packets"

- Check logs: journalctl -u mypicamera.service -f

- Check Laptop Firewall: This is the most common culprit. Temporarily disable your Windows/Mac firewall to see if the packets start arriving in Unity.

- Verify IP: Open a terminal on your laptop and type ipconfig. Ensure your IP is in the 192.168.50.x range (or whatever range the Pi is handing out).

The Unity "Bridge" (C#)

Unity runs on a single thread. If you tell Unity to "listen" for a network packet, the whole game will freeze until a packet arrives. To fix this, we use a Thread to listen in the background and a Queue or a simple variable to update the boat.

Create a new C# script called UDPReceiver.cs and attach it to an empty GameObject:

C#

using UnityEngine;

using System.Net;

using System.Net.Sockets;

using System.Text;

using System.Threading;

public class UDPReceiver : MonoBehaviour

{

Thread receiveThread;

UdpClient client;

public int port = 5005;

public string lastCommand = "";

void Start() {

receiveThread = new Thread(new ThreadStart(ReceiveData));

receiveThread.IsBackground = true;

receiveThread.Start();

}

private void ReceiveData() {

client = new UdpClient(port);

while (true) {

try {

IPEndPoint anyIP = new IPEndPoint(IPAddress.Any, 0);

byte[] data = client.Receive(ref anyIP);

lastCommand = Encoding.UTF8.GetString(data);

Debug.Log("Received: " + lastCommand);

} catch (System.Exception e) {

Debug.Log(e.ToString());

}

}

}

// This runs every frame and moves the boat based on the last command received

void Update() {

if (lastCommand == "FORWARD") {

transform.Translate(Vector3.forward * Time.deltaTime * 5f);

} else if (lastCommand == "LEFT") {

transform.Rotate(Vector3.up, -1f);

} else if (lastCommand == "RIGHT") {

transform.Rotate(Vector3.up, 1f);

}

}

void OnApplicationQuit() {

if (receiveThread != null) receiveThread.Abort();

client.Close();

}

}